# RadCam Camera Settings Reference

This document lists the camera settings DORIS can read and apply on the RadCam (Blue Robotics 4K “br4kcam”). DORIS does not talk to the camera directly; it forwards each setting to the radcam-manager extension, which speaks the camera's native HTTP protocol.

Settings are grouped by the manager's three protocol groups:

| Group | Read / Write action | Model |
| --- | --- | --- |
| Video / Encoding | getVencConf / setVencConf | VideoParameterSettings |
| Base image | getImageAdjustment / setImageAdjustment | BaseParameterSetting |
| Advanced image | getImageAdjustmentEx / setImageAdjustmentEx | AdvancedParameterSetting |

Notes:

The native key (shown in parentheses) is the exact JSON key the camera expects, which is what preset files store.

Value ranges and enumerations are taken from the radcam-manager protocol definitions (br4kcam_commands).

Enable/disable toggles use the camera's wording: Open = enabled, Close = disabled. The numeric value differs per setting (e.g. Mirror, Flip and Lens Correction use Open = 0; most other toggles use Close = 0), so check each entry rather than assuming 0 = disabled.

Fields marked [read-only] are reported by the camera but cannot be set.

Some settings are only honored when a related setting has a particular value; those dependencies are noted under Values.

Not every setting is meant for end users; the full list is exposed now for experimentation and will be trimmed to a practical subset later.

DORIS defaults: the settings in sections H (Day/Night & IR-Cut), I (Light / IR LED), J (Aperture / Iris) and M (Scene Mode) have hardware-appropriate defaults supplied by DORIS. These are fill-if-missing defaults, not hard overrides: DORIS fills them in only when a preset/apply doesn't specify them, and applies them as a baseline at startup / dive start when no preset is active. An advanced user can still override any of them by including the key in a preset or hand-edited JSON — the explicit value is honored. They are simply hidden from the experimentation panel because there's no dedicated control for them yet (their values are still stored). The default values are: color_black=0 (color/day, IR-cut filter engaged, never night mode), led_control=2 (illuminator disabled — no camera-controlled LEDs), auto_iris=1 (iris disabled — fixed-aperture lens), scene_mode=0 and sceneMode=0 (plain general video capture).

# A. Video & Encoding

1. Channel (channel) — Which stream to configure.

Values: 0 Main, 1 Auxiliary, 2 Third (enum)

2. Encode Profile (encode_profile) — H.264/H.265 profile; higher compresses more efficiently at the cost of decode complexity.

Values: 0 Baseline, 1 Main, 2 High (enum)

3. Codec (encode_type) — Video codec. H.265 gives roughly the same quality at about half the bitrate.

Values: 1 = H.264, 5 = H.265 (enum)

4. Supported Resolutions (pixel_list) [read-only] — The width x height combinations the camera supports.

Values: read-only list of {width, height} pairs (e.g. 2560x1440, 1920x1080)

5. Picture Width (pic_width) — Encoded frame width; must match a width offered in pixel_list.

Values: integer pixels from pixel_list (e.g. 2560, 1920)

6. Picture Height (pic_height) — Encoded frame height; must match a height offered in pixel_list.

Values: integer pixels from pixel_list (e.g. 1440, 1080)

7. Bitrate Mode (rc_mode) — VBR (Variable) is quality-priority; CBR (Constant) gives predictable bandwidth/file size.

Values: 0 VBR, 1 CBR (enum)

8. Bitrate (bitrate) — Target bitrate; higher = better quality and larger files.

Values: integer, kbps (e.g. 6144)

9. Max Frame Rate (max_framerate) [read-only] — Highest frame rate the camera supports at the current resolution.

Values: read-only integer fps (e.g. 25)

10. Frame Rate (frame_rate) — Encoded frames per second.

Values: integer fps, up to max_framerate (e.g. 25)

11. GOP / I-frame Interval (gop) — Frames between keyframes; lower = faster seeking/recovery, larger files.

Values: integer, frames (e.g. 50)

# B. Image Tone & Color

12. Hue (hue) — Color tint / shift.

Values: integer 0-255

13. Brightness (brightness) — Overall image lightness.

Values: integer 0-255

14. Sharpness (sharpness) — Edge enhancement / apparent detail.

Values: integer 0-255

15. Contrast (contrast) — Difference between light and dark areas.

Values: integer 0-255

16. Saturation (saturation) — Color intensity.

Values: integer 0-255

17. Gamma (gamma) — Midtone response / tone curve shape.

Values: integer 0-255

# C. Exposure

18. Backlight Compensation (blc_level) — Brightens a subject lit from behind.

Values: integer 0-255

19. Max Exposure (max_exposure) — Longest exposure the auto-exposure loop may use, as the x in T = 1/x seconds (larger x = shorter max exposure).

Values: enum: 12, 25, 30, 50, 60, 100, 200, 400, 800, 1000, 2000, 4000, 8000

20. Auto Exposure Mode (auto_exposureEx) — Auto or manual exposure.

Values: 0 Auto, 1 Manual (enum)

21. Exposure Strategy (AE_strategy_mode) — Metering priority in auto exposure.

Values: 0 Highlight priority, 1 Lowlight priority (enum)

22. Manual Exposure Time (exposure_time) — Fixed shutter time as the x in T = 1/x seconds; used only in manual exposure.

Values: enum: 12, 25, 30, 50, 60, 100, 200, 400, 800, 1000, 2000, 4000, 8000, 10000, 34464

# D. White Balance

23. Auto White Balance (auto_awb) — Auto or manual white balance.

Values: 0 Auto, 1 Manual (enum)

24. WB Red Gain (awb_red) — Manual red channel gain (manual mode).

Values: integer 0-255

25. WB Green Gain (awb_green) — Manual green channel gain (manual mode).

Values: integer 0-255

26. WB Blue Gain (awb_blue) — Manual blue channel gain (manual mode).

Values: integer 0-255

27. WB Scene (awb_auto_mode) — Auto white-balance scene preset.

Values: 0 Scene1, 1 Scene2 (enum)

28. WB Style Red (awb_style_red) — Red bias applied on top of the auto result.

Values: integer 0-255

29. WB Style Green (awb_style_green) — Green bias applied on top of the auto result.

Values: integer 0-255

30. WB Style Blue (awb_style_blue) — Blue bias applied on top of the auto result.

Values: integer 0-255

31. One-Push Auto White Balance (onceAWB) [Advanced group] — Runs a single white-balance convergence against the current scene, then holds it. This is what the DORIS “Auto White Balance on Lights” toggle fires once the bottom lights come on.

Values: 0 or 1 (write 1 to trigger)

# E. Gain / ISO

32. Auto Gain Mode (auto_gain_mode) — Auto or manual gain.

Values: 0 Auto, 1 Manual (enum)

33. Max Auto Digital Gain (auto_DGain_max) — Ceiling for digital gain in auto mode.

Values: integer 0-255

34. Max Auto Analog Gain (auto_AGain_max) — Ceiling for analog (sensor) gain in auto mode.

Values: integer 0-255

35. Max System Gain (max_sys_gain) — Overall gain ceiling.

Values: integer 0-255

36. Manual Analog Gain Enable (manual_AGain_enable) — Enable a fixed analog gain.

Values: 0 Close (disabled), 1 Open (enabled) (enum)

37. Manual Analog Gain (manual_AGain) — Fixed analog gain value.

Values: integer 0-255

38. Manual Digital Gain Enable (manual_DGain_enable) — Enable a fixed digital gain.

Values: 0 Close (disabled), 1 Open (enabled) (enum)

39. Manual Digital Gain (manual_DGain) — Fixed digital gain value.

Values: integer 0-255

# F. Wide Dynamic Range & Highlight

40. WDR Level (digital) (wdr_level) — Digital wide-dynamic-range strength (balances bright and dark regions).

Values: integer 0-255

41. WDR Sensor Enable (wdr_sensor) — Enable true sensor-based WDR.

Values: 0 Close (disabled), 1 Open (enabled) (enum)

42. WDR Sensor Level (wdr_level_sensor) — Strength of the sensor-based WDR.

Values: integer 0-255

43. HLC Enable (hlc_enable) — Highlight compensation; dims very bright spots (e.g. direct light sources).

Values: 0 Close (disabled), 1 Open (enabled) (enum)

# G. Noise Reduction & Enhancement

44. Noise Reduction (3D) (noiseReduction) — Temporal (3D) noise reduction.

Values: 0 Close (disabled), 1 Low, 2 Middle, 3 High (enum)

45. 2D NR Level (_2DNR_level) — Spatial (single-frame) noise reduction strength.

Values: 0 Low, 1 Middle, 2 High (enum)

46. Lens Correction (lens_correction) — Corrects geometric lens distortion.

Values: 0 Open (enabled), 1 Close (disabled) (enum)

47. Anti-Fog (antiFog) [Base group] — Defog / dehaze enhancement.

Values: 0 Close (disabled), 1 Open (enabled) (enum)

48. Frame Turbo Pro (frameTurbo_pro) [Base group] — Frame-rate enhancement / turbo processing.

Values: 0 Close (disabled), 1 High frame rate, 2 Ultra-high frame rate (enum)

# H. Day / Night & IR-Cut

DORIS default: color_black=0 (color/day; IR-cut filter engaged; never night mode) unless a preset/JSON overrides it. The remaining entries in this section only matter in auto/night modes.

49. Color / Black Mode (color_black) — Force color, or auto-switch to black-and-white at night.

Values: 0 Color, 1 Auto (enum)

50. IR Detect Mode (infr_detect_mode) — How day/night is decided.

Values: 0 VideoDetection, 1 TimeControl, 2 PhotosensitiveDetection (enum; only when color_black = 1)

51. Sensitivity Day->Night (sens_day_to_night) — Threshold for switching into night mode.

Values: integer 0-255 (only when infr_detect_mode = 0)

52. Sensitivity Night->Day (sens_night_to_day) — Threshold for switching back to day mode.

Values: integer 0-255 (only when infr_detect_mode = 0)

53. IR Day Hour (infr_day_h) — Scheduled day-start hour (TimeControl mode).

Values: integer 0-23 (only when infr_detect_mode = 1)

54. IR Day Minute (infr_day_m) — Scheduled day-start minute.

Values: integer 0-59 (only when infr_detect_mode = 1)

55. IR Night Hour (infr_night_h) — Scheduled night-start hour.

Values: integer 0-23 (only when infr_detect_mode = 1)

56. IR Night Minute (infr_night_m) — Scheduled night-start minute.

Values: integer 0-59 (only when infr_detect_mode = 1)

57. IR-Cut Level (ircut_level) — IR-cut filter level.

Values: 0 Low, 1 High (enum)

58. LDR / Photosensitive Level (ldr_level) — Ambient light (photoresistor) level.

Values: 0 Low, 1 High (enum)

# I. Light / IR LED Control

DORIS default: led_control=2 (illuminator disabled) unless a preset/JSON overrides it. This vehicle has no camera-controlled LEDs, so the rest of this section is normally unused.

59. LED Control Mode (led_control_mode) — How the illuminator is driven.

Values: 0 Electrical level, 1 PWM (enum)

60. Lamp Type (lamp_type) — Illuminator type.

Values: 0 Infrared, 1 White light, 2 Auto (enum)

61. LED Enable Level (led_control_avail) — Electrical-level enable for the light.

Values: 0 Low, 1 High (enum; only when led_control_mode = 0)

62. IR Level (ir_level) — Infrared illuminator brightness.

Values: integer 0-255 (only when led_control_mode = 1)

63. LED Level (led_level) — White-light brightness.

Values: integer 0-255 (only when lamp_type = 1 and led_control_mode = 1)

64. LED / IR Control (led_control) — Illuminator enable/disable/auto.

Values: 0 Auto, 1 Open (enabled), 2 Close (disabled) (enum)

# J. Aperture / Iris

DORIS default: auto_iris=1 (iris disabled) unless a preset/JSON overrides it. This camera has a fixed-aperture lens, so Iris Level is never reported as settable.

65. Auto Iris (auto_iris) — Aperture mode.

Values: 0 Open (auto enabled), 1 Close (disabled), 2 Manual (enum)

66. Iris Level (irisLevel) [read-only] — Aperture PWM duty cycle.

Values: read-only integer 0-255 (reported only when auto_iris = 2; not settable)

# K. Shutter & Flicker

67. Slow Shutter Mode (low_farme_rate) — Enable slow shutter for low-light scenes (the camera spells this key low_farme_rate).

Values: 0 Close (disabled), 1 Open (enabled) (enum)

68. Anti-Flicker (anti_flicker) — Reduces flicker under artificial light.

Values: 0 Close (disabled), 1 Auto, 2 50 Hz, 3 60 Hz (enum)

69. Power Frequency (power_freq) — Video/mains standard.

Values: 0 NTSC, 1 PAL (enum)

# L. Orientation

70. Rotate (rotate) [Base group] — Image rotation.

Values: 0 = 0°, 1 = 90°, 2 = 180°, 3 = 270° (enum)

71. Mirror (mirror) [Advanced group] — Horizontal flip.

Values: 0 Open (enabled), 1 Close (disabled) (enum)

72. Flip (flip) [Advanced group] — Vertical flip.

Values: 0 Open (enabled), 1 Close (disabled) (enum)

# M. Scene Mode

DORIS default: advanced scene_mode=0 (IPC / general video) and base sceneMode=0 unless a preset/JSON overrides them. DORIS is normally used only for general video capture, so the face/plate profiles are not used.

73. Scene Mode (Base) (sceneMode) [Base group] — Reportedly non-functional on this camera; prefer the advanced scene mode below.

Values: 0 Off, 1 FaceCapture, 2 LicensePlateCapture (enum)

74. Scene Mode (Advanced) (scene_mode) [Advanced group] — Capture scene profile.

Values: 0 IPC, 1 FaceCapture, 2 LicensePlateCapture (enum)

# N. Restore Defaults

75. Restore Base Defaults (set_default) [Base group] — Reset all base image settings to factory defaults.

Values: write 1 to reset

76. Restore Advanced Defaults (set_default) [Advanced group] — Reset all advanced image settings to factory defaults.

Values: write 1 to reset

# Related DORIS automation

Auto White Balance on Lights (Descent, On Bottom and Ascent phases) — Each phase has its own toggle in the Camera section. When enabled, the dive script fires the one-push AWB (onceAWB, #31) a couple seconds after that phase's lights first turn on, so white balance is calibrated for the lit scene rather than ambient surface light. If the phase's lights are disabled, the AWB fires immediately when the phase begins.

Presets — Any combination of the settings above can be saved to a named preset. The active preset is auto-applied at DORIS startup and at dive start, and presets can be downloaded/imported as JSON using these same native keys.